EC204

Digital System Design Lab

Lab – 2

Utkarsh R Mahajan

201EC164, S11

1] Half adder (a) using gates (b) using dataflow model (using assign statement)

(a) using gates

module halfAdder(input a,input b,output sum,output carry);

xor xor1(sum, a, b);

and and1(carry, a, b);

endmodule

(b) using dataflow model

module halfAdderdf(input a,b,output sum, carry);

assign sum = a^b;

assign carry = a&b;

endmodule

2] Full adder (a) using dataflow model (using assign statement) (b) using two half adders and an OR gate

(a) using dataflow model

module fullAdderdf(input a, b, carryin,output sum, carryout);

assign sum = carryin^(a^b);

assign carryout = (a&b) | carryin&(a^b);

endmodule

(b) using two half adders and an OR gate

module HalfAdderdf(input a,b,output sum, carry);

assign sum = a^b;

assign carry = a&b;

endmodule

module FullAdder2haor(input a, b, carryin,output sum, carryout);

wire m2b, m1co, m2co;

HalfAdderdf m1 (.a(a),.b(b),.sum(m2b),.carry(m1co));

HalfAdderdf m2 (.a(carryin),.b(m2b),.sum(sum),.carry(m2co));

assign y = m1co | m2co;

endmodule

3] Two to one multiplexer using behavioral model (using if statement)

module Mux_2_1(input A, B, S0, output reg Y);

always @(*)

begin

if(S0==1) Y =A;

else Y = B;

end

endmodule

4] 4 bit ripple carry adder using full adder modules.

module FullAdder(input A, B, Ci,output S, Co);

assign S = Ci^(A^B);

assign Co = (A&B) | Ci&(A^B);

endmodule

module FourBitAdder(input [3:0]A, B,

input Ci,

output [3:0]S,

output Co);

wire  C1, C2,C3;

FullAdder FA1(.A(A[0]),.B(B[0]),.Ci(Ci),.S(S[0]),.Co(C1));

FullAdder FA2(.A(A[1]),.B(B[1]),.Ci(C1),.S(S[1]),.Co(c2));

FullAdder FA3(.A(A[2]),.B(B[2]),.Ci(c2),.S(S[2]),.Co(c3));

FullAdder FA4(.A(A[3]),.B(B[3]),.Ci(c3),.S(S[3]),.Co(Co));

endmodule

module Test4bitadder();

/*201ec164*/

reg  [1:0] ci ;

integer ca, cb;/* ca and cb are counts*/

wire Cout;

wire [3:0] s;

reg C;

reg [3:0] A, B;

reg [3:0] array[0:15];

assign array[0] = 4'b0000; assign array[1] = 4'b0001;

assign array[2] = 4'b0010; assign array[3] = 4'b0011;

assign array[4] = 4'b0100; assign array[5] = 4'b0101;

assign array[6] = 4'b0110; assign array[7] = 4'b0111;

assign array[8] = 4'b1000; assign array[9] = 4'b1001;

assign array[10] = 4'b1010; assign array[11] = 4'b1011;

assign array[12] = 4'b1100; assign array[13] = 4'b1101;

assign array[14] = 4'b1110; assign array[15] = 4'b1111;

assign C=0;

FourBitAdder test(.A(A), .B(B), .Ci(C), .S(s), .Co(Cout));

initial

begin   for( ca =0; ca<16; ca=ca+1)

begin   for(cb=0; cb<16; cb=cb+1)

begin

A=array[ca];

B=array[cb];

#100;

end

end

end

endmodule

5] A four variable logic function that is equal to 1 if any three or all four of its variables are equal to 1 is called a majority function. Write a Verilog code that implements this majority function. Use the Boolean equation derived in Lab1 in assign statement.

module Majority4Var(input A, B, C, D, output Y);

assign Y =  ~((~(A|B))|(~(A|C))|(~(A|D))|(~(B|C))|(~(B|D))|(~(C|D)));

endmodule

/*201EC164*/

module Majority4VarTest();

reg A, B, C, D;

reg [1:0] inp;

integer a,b,c,d;

assign inp[0] =0;

assign inp[1] =1;

wire Y;

Major4B test(.A(A), .B(B), .C(C), .D(D), .Y(Y));

initial

begin

for(a=0;a<2;a=a+1)

begin   for(b=0;b<2;b=b+1)

begin   for(c=0;c<2;c=c+1)

begin   for(d=0;d<2;d=d+1)

begin   A=inp[a]; B=inp[b];

C=inp[c]; D=inp[d];

#100;

end

end

end

end

end

endmodule