EC201

Analog Electronics Lab

Lab – 1

Utkarsh R Mahajan 201EC164

Sannan Ali 201EC159

Experiment 1: Logic Gates using Diodes

Aim: To design OR and AND gates using diodes and resistors for:

A) Positive logic

B) Negative logic

1.a] Positive Logic OR Gate

Output is high whenever any one of the inputs, or both the inputs are high. Circuit diagram:

Circuit in LTspice:

Case 1: V1=0V, V2=0V

Both the inputs are low So the expected output is also low.

now Simulation in LTspice:

As we can see, the output voltage Vout is 0V, which is low and is the expected output.

Case 2: V1=0V, V2=5V or V1=0V, V2=5V

One of the inputs is high, hence the expected output is also high. Simulation in LTspice:

As we can see, the output is 4.3V, which is expected because the Diode has a potential of its own, and we can conclude that our Vout is also high.

Case 3: V1=5V, V2=5V

Since both inputs are high, The expected output is high.

Simulation in LTspice:

Results:

| V1 | V2 | Vout | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 5 | 4.3 | 1 |
| 5 | 0 | 4.3 | 1 |
| 5 | 5 | 4.3 | 1 |

b] Negative Logic OR Gate

Output is high whenever any one of the inputs, or both the inputs are high. Circuit diagram:

Circuit in LTspice:

Case 1: V1=0V, V2=0V

Both the inputs are low So the expected output is also now Simulation in LTspice:

As we can see, the output voltage Vout is 0V, which is low and is the expected output.

Case 2: V1=0V, V2=-5V or V1=-5V, V2=0V

One of the inputs is high, hence the expected output is also high. Simulation in LTspice:

As we can see, the output is -4.3V, which is expected because the Diode has a potential of its own, and we can conclude that our Vout is also high.

Case 3: V1=-5V, V2=-5V

Since both inputs are high, The expected output is high.

Simulation in LTspice:

Results:

| V1 | V2 | Vout | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | -5 | -4.3 | 1 |
| -5 | 0 | -4.3 | 1 |
| -5 | -5 | -4.3 | 1 |

c] Positive Logic AND Gate

Output is high whenever both of the inputs are high.

Circuit diagram:

Circuit in LTspice:

Case 1: V1=0V, V2=0V

Both the inputs are low So the expected output is also low.

now Simulation in LTspice:

As we can see, the output voltage Vout is 0V, which is low and is the expected output.

Case 2: V1=0V, V2=5V or V1=5V, V2=0V

Only one of the inputs is high, hence the expected output is low. Simulation in LTspice:

As we can see, the output voltage Vout is 0V, which is low and is the expected output

Case 3: V1=5V, V2=5V

Since both inputs are high, the expected output is high.

Simulation in LTspice:

As we can see, the output voltage Vout is high and is the expected output.

Results:

| V1 | V2 | Vout | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 5 | 0 | 0 |
| 5 | 0 | 0 | 0 |
| 5 | 5 | 4.3 | 1 |

2] Positive Logic AND Gate

Output is high whenever both of the inputs are high.

Circuit diagram in LTspice:

As we can see, the output voltage Vout is 0V, which is low and is the expected output.

Case 2: V1=0V, V2=-5V or V1=-5V, V2=0V

Only one of the inputs is high, hence the expected output is low. Simulation in LTspice:

As we can see, the output voltage Vout is 0V, which is low and is the expected output

Case 3: V1=5V, V2=5V

Since both inputs are high, the expected output is high.

Simulation in LTspice:

As we can see, the output voltage Vout is high and is the expected output.

Results:

| V1 | V2 | Vout | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | -5 | 0 | 0 |
| -5 | 0 | 0 | 0 |
| -5 | -5 | -4.3 | 1 |

2: List/Explain the following

1] Typical voltage drop across the diode (Silicon, Germanium and Gallium Arsenide) for 10mA of forward current.

| Diode | Voltage Drop |
| --- | --- |
| Silicon (Si) | 0.7V |
| Germanium (Ge) | 0.3V |
| Gallium Arsenide (GaAs) | 1.2V |

Since the voltage drop in case of forward bias depends upon the material of the diode, it is independent of the forward current.

2] typical reverse saturation current in 1N4001 diode

| TA | Reverse Saturation Current | Units |
| --- | --- | --- |
| 25 °C | 5 | µA |
| 125 °C | 50 | µA |

We can see that the reverse saturation current increases with increase in temperature because it leads to more electron-hole pairs which increase the conductivity.