EC-210

Microprocessors Lab

LAB-4

Utkarsh Mahajan 201EC164

ARNAV RAJ 201EC109

Objective: To study defining memory area, constant in the assembly program

Exercise:

4.2] Reverse the string and check if the string is a palindrome.

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing reversed string

str_rev SPACE 1024;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;Pointer to the input string

LDR R1, =str_src ;

;Pointer to the reversed string

LDR R2, =str_rev ;

;storing the input's pointer in R8

ADD R8, R1, #0;

;Storing the result's pointer in R9

ADD R9, R2, #0;

; load a byte from input string and update the pointer

lth     LDRB R3, [R8],#1;

; counting length (total = length +1)

ADD R4, #1;

; Check for End of string

CMP R3, #0 ;

; If not go back to lth to count length

BNE lth;

; total = (length +1) - 1

SUB R4, #1;

;storing length in R5

ADD R5, R4, #0;

; R8 to point to the last value just before the 0 (NULL).

SUB R8, #2;

; for iteration count R5 = R5 -1;

res SUB R5, #1;

; Loading the value in reverse order in R3

LDRB R3, [R8],#-1 ; load a byte and update the pointer

; Storing the loaded value;

STRB R3, [R9],#1 ; store byte and update the pointer

; checking if R5 is 0. which indicated we are done reversing.

CMP R5, #0 ; Check for End of string

; If its not then loop again.

BNE res

; storing null in R3

MOV R3, #0;

; adding NULL at the end of the reversed string

STRB R3, [R9];

;Restoring length in R5

ADD R5, R4, #0;

;storing starting addresses of input & reversed string.

ADD R8, R1, #0;

ADD R9, R2, #0;

;checking if the input string is palindrome

; by comparing same index bits of input and reversed string.

pal LDRB R3, [R8],#1 ;

LDRB R4, [R9],#1 ;

; checking for end of string

CMP R3, #0;

; As once it reaches end means it's a palindome.

BEQ ipa;

;comparing bytes.

CMP R3, R4;

; if equal -> it can be a palindrome hence looping for next index

BEQ pal;

; If not equan then its not a palindrome

;storing 0 in R0 (output register)

np      MOV R0, #0;

; jump to stop;

BAL stop;

;storing 1 in R0 (output register) if input is a palindrome

ipa     MOV R0, #1;

BAL stop;

stop BAL stop

; input_string

str_src DCB "reviver",0;

NOP

END

Debugging:

Initial Memory: (after getting the address through register)

Memory 1 shows input memory and memory 2 shows output(reversed)

Setup:

Final Output:

Final Register Values:

Final Memory:

Observation: The output of if the string was palindrome is stored in R0. We can see that its 1 and our input string is actually also an palindrome. We can also see the output memory in memory2. It is reverse of the string that we entered.

For input string: "random text"

Output register:

Output memory:

Observation: we can see that R0 is indicating the input string is not a palindrome and we can also see the reversed string in output memory (2).

4.3] Find the substring in Main string

->

Source Code:

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

; Pointer to the input string

LDR R1, =str_src ;

; Pointer to the input substring

LDR R2, =sub_str;

;storing the input string's pointer in R8

ADD R8, R1, #0;

;storing the input substring's pointer in R9

ADD R9, R2, #0;

; Loading the first byte(char) of the substring

LDRB R4, [R9],#1 ;

; for storing the index.

MOV R5, #0;

;loading the chars of input string.

ind     LDRB R3, [R8],#1 ;

; incrementing the index.

ADD R5, #1;

; Check for End of string

CMP R3, #0 ; Check for End of string

; if it reaches the end of string -> not found

BEQ nf;

;comparing 1st char of substring and i'th char of string

CMP R3, R4;

;back to iteration if not equal.

BNE ind;

; else

; storing index in R10 for inner loop.

ADD R10, R8, #0;

; comparing subsequent characters

ver LDRB R3, [R10],#1 ; load a byte and update the pointer

LDRB R4, [R9],#1 ; load a byte and update the pointer

CMP R4, #0 ; Check for End of sub string

; if entire substring satisfies, we return.

BEQ iss;

;Check if char's are equal

CMP R3, R4 ;

;If not equal, point R9 to start of string and reload R4

ADDNE R9, R2, #0;

LDRBNE R4, [R9],#1 ;

;back to main iteration if not equal.

BNE ind;

; if current chars are equal loop further.

BEQ ver

;storing -1 if substring not found.

nf      MOV R0, #-1;

BAL stop;

; storing index of substring in R0 if substring found.

iss     ADD R0, R5, #-1;

BAL stop;

stop BAL stop

; input string

str_src DCB "reviver",0;

; input substring

sub_str DCB "vive",0;

NOP

END

Debugging:

Initial Memory: (after getting the address through register)

Memory 1 shows input string and memory 2 shows input substring

Setup:

Final Output:

Final Register Values:

Observation: We can see that the output stored in R0 is 2. Which is correct as our input string a substring were “reviver” and “vive” respectively from which we can see that vive starts at index 2 (counting from 0).

4.4] Insert a substring in main string at given position.

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing the final string

str_rev SPACE 1024

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;Pointer to the input string

LDR R1, =str_src ;

;Pointer to the substring

LDR R2, =sub_str ;

;Pointer to result string

LDR R12, =str_rev ;

;Pointer to index

LDR R11, =sub_index ;

;loading index in R10;

LDRB R10, [R11];

;storing the input string's pointer in R8

ADD R8, R1, #0;

;storing the input substring's pointer in R9

ADD R9, R2, #0;

;for iteration till the insertion index

MOV R7, #0;

;result's pointer in R12.

ADD R6, R12, #0;

;storing the prefix part of string in result(before the insertion index)

;load a byte and update the pointer

pref LDRB R3, [R8],#1 ;

;store byte and update the pointer

STRB R3, [R6], #1;

;incrementign R7.

ADD R7, #1;

;check if the insertion index is reached.

CMP R7, R10;

; if not then loop. again to pref.

BNE pref

; storing the substring in consequent index in result.

; load a byte and update the pointer

sbst LDRB R3, [R9],#1 ;

;store byte and update the pointer

STRB R3, [R6], #1;

;Check for End of string

CMP R3, #0 ;

; If not, then loop.

BNE sbst

; subtract result pointer, so the null value can be rewritten

SUB R6, #1;

;storing the suffix part, the part that existed at right of insertion index

;load a byte and update the pointer

sufx LDRB R3, [R8],#1 ;

; store byte and update the pointer

STRB R3, [R6], #1;

; Check for End of string

CMP R3, #0 ;

; if not , loop again

BNE sufx

done MOV R0, #1;

BAL stop;

stop BAL stop

;input string

str_src DCB "reviver",0;

;input substring

sub_str DCB "abc",0;

;input insertion index

sub_index DCB 2;

NOP

END

Debugging:

Initial Memory: (after getting the address through register)

Memory 1 shows input string, memory 2 input substring and memory 3 will hold output result string.

Index:

Setup:

Final Output:

Final Memory:

Observation: We can see that in memory3, we have our final string. We can compare both the input string and substring with that of in memory 3 and verify that it contains the string with substring at insertion index.

4.5] Squeeze a string removing all the blank spaces and store it in the same location

-> We will copy the input string to the space. And then we will operate on it. This way we can directly operate on the same memory.

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for the input string

str_main SPACE 1024

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;Pointer to the string

LDR R1, =str_main ;

;Pointer to the input string

LDR R2, =str_src ;

;storing the string's pointer in R8

ADD R4, R1, #0;

;storing the input's pointer in R8

ADD R5, R2, #0;

;for storing the input String;

; loading characters from input string.

fill LDRB R3, [R5], #1;

;storing the character in result space

STRB R3, [R4], #1; store byte and update the pointer

;checking if end of line is reached

CMP R3, #0;

; if not then loop.

BNE fill;

; for storing no of zeroes encountered

MOV R4, #0;

; Pointer to the string in R5

MOV R5, R1;

;loading character from string

sqz LDRB R3, [R5];

; checking if its a space

CMP R3, #32;

; if its a space then add 1 to R4.

ADDEQ R4, #1;

; if not then calculate index to move towards left.

SUBNE R6, R5, R4;

; and also store the current value towards left at that index.

STRBNE R3, [R6];

; update the string's iterator

ADD R5, #1;

;check if string ends.

CMP R3, #0;

; if not then loop.

BNE sqz;

;R0 to change to 1 at the end of the loop.

done MOV R0, #1;

BAL stop;

stop BAL stop;

str_src DCB "This is a text",0;

NOP

END

Initial Memory: (after getting the address through register and loading the value into the reserved space)

Memory 1 will hold the input string and later also the result.

Setup:

Final Output:

Final Memory:

Observation: We can see that the final string is squeezed and all spaces are removed.