EC-210

Microprocessors Lab

LAB-7

Utkarsh Mahajan 201EC164

ARNAV RAJ 201EC109

Objective: To understand division and code conversion operations.

Exercise:

7.1] Write a program to divide

(a) 32 bit number by 16 bit number.

->

Repeated subtraction:

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

c SPACE 8;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =a;

LDR R2, =bi;

;address to the result

LDR R3, =c ;

LDR R4, [R1]; load a

LDR R5, [R2]; load b

MOV R6, #0;

div CMP R4, R5; ? a < b

BCC done; J if C clear (a<b)

SUB R4, R4, R5; a=a-b;

ADD R6, R6, #1; Q = Q+1;

B   div; J to div for loop

done STR R6, [R3]; storing quotient

STR R4, [R3, #4]; storing remainder

stop BAL stop

; input_number

a   DCD 0xFA124025;

bi  DCD 0xCF85;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Inputs: a= 0xFA124025(4195500069), b=0xCF85(53125)

Expected results: 78974.1189459

Which is Q= 78974, R=6319

Final Output:

Final Memory: (in decimal)

Comparing the final memory with expected result, we can see that our code is correct.

shift & subtract Method:

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

c SPACE 8;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =a;

LDR R2, =bi;

;address to the result

LDR R3, =c ;

LDR R4, [R1]; load a

LDR R5, [R2]; load b

MOV R6, #1;

div CMP R5, #0x80000000;

CMPCC R5, R4;

LSLCC R5, R5, #1;

LSLCC R6, R6, #1;

BCC div;

MOV R7, #0;

div2 CMP R4, R5;

SUBCS R4, R4, R5;

ADDCS R7, R7, R6;

LSRS R6, R6, #1;

LSRNE R5, R5, #1;

BNE div2;

STR R7, [R3],#4;

STR R5, [R3];

stop BAL stop

; input_number

a   DCD 0xFA124025;

bi  DCD 0xCF85;

END

Final Output:

It matches the final output.

Execution Time:

Subtraction method: 0.00000015sec

Shift and divide: 0.00000453sec

For smaller values, shift and divide seems to take more time.

(b) 64 bit number by 32 bit number

->

Repeated subtraction:

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

c SPACE 8;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =a;

LDR R2, =bi;

;address to the result

LDR R3, =c ;

LDR R4, [R1]; load a_low

LDR R5, [R1, #4]; load a_high

LDR R6, [R2]; load b

MOV R7, #0; Q=0;

div CMP R5, #0;div process while a_high>0

BEQ dlow;

SUBS R4, R4, R6; R4=R4-R6

SUBCC R5, R5, #1; Sub borrow(if) from a_high.

ADD R7, R7, #1; Q = Q+1;

B   div

dlow CMP R4, R6; Now Normal Subtraction process

BCC done; for division like before

SUB R4, R4, R6;  R4=R4-R6

ADD R7, R7, #1; Q=Q+1

B   dlow;

done STR R7, [R3]; storing quotient

STR R4, [R3, #4]; storing remainder

stop BAL stop

; input_number a=0x703D4444FA124025

; b= 0xFEFF4ED6

a   DCD 0xFA124025, 0x703D4444;

bi  DCD 0xFEFF4ED6;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory:

0x703D4444FA124025/  0xFEFF4ED6 = 8087695568871243813/ 4278144726

Comparing the results with an online precise calculator: (https://keisan.casio.com/calculator)

We can see that our quotient matches.

Now for the remainder:

We can see that even it matches. Hence our result is correct.

7.2] Write a program that takes a 16 bit Hex number and converts it into its BCD equivalent.

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for result

array SPACE 1024;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =inputnumber ;

;address for result

LDR R2, =array;

MOV R12, R2;

; load the  input number from memory

LDR R4, [R1];

; Getting the last bit

MOV R5, R4; R5 = R4

MOV R6, #0;

MOV R7, #0; for storing no of nos.

;dividing and finding nth digit.

;division by the use of subtracting

; until a remainder is found.

; where remainder < 10

div SUB R5, R5, #10; subtracting by 10

CMP R5, #10; checking if remainder < 10

;incrementing iterator to store quotient

ADD R6, R6, #1;

; if remainder >=10 then loop to sub further.

BPL div

ADD R7, #1; incrementing nos.

STRB R5, [R2], #1; storing digits

;comparing quotient and 10.

CMP R6, #10;

;if quotient >10 then store quotient in remainder

MOVPL R5, R6;

; && reset quotient

MOVPL R6, #0;

; && loop again

BPL div;

; check if quotient is zero

CMP R6, #0;

; increment no of digits counter if no

ADDNE R7, #1;

; &&  store it in R2 if no.

STRBNE R6, [R2]; storing last digit

;

stop BAL stop

; input_number

inputnumber DCD 0xFABC;64188

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory:

We can see that for our input 0xFABC (64188 in decimal) the BCD values stored matches.

7.3] Write a program that takes an 4 digit BCD number and converts it into is Hex equivalent.

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

bcd SPACE 4;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =asc;

;address to the result

LDR R3, =bcd ;

MOV R8, #1; multiplier for nth digit

MOV R7, #0;

MOV R10, #10;

LDRB R5, [R1, #3]; load digit

ADD R7, R7, R5;

MUL R9, R8, R10;

LDRB R5, [R1, #2]; load digit

MUL R7, R10, R7;

ADD R7, R7, R5;

LDRB R5, [R1, #1]; load digit

MUL R7, R10, R7;

ADD R7, R7, R5;

LDRB R5, [R1]; load digit

MUL R7, R10, R7;

ADD R7, R7, R5;

STR R7, [R3]; storing quotient

stop BAL stop

; input_number

asc DCB 2, 1, 2, 1;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory:

We can see that the output matches our input 1212 which was in BCD format.

7.4] Perform addition of two 8 digit BCD numbers and give the result in BCD.

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Utkarsh && Arnav Lab6

;Space for storing result

z SPACE 9;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =x;

LDR R2, =y;

;address to the result

LDR R3, =z ;

MOV R0, #0;

MOV R7, #0; carry_bit xD

loop CMP R0, #8;

BEQ done; jump if all bits done

LDRB R4, [R1], #1; load bit, x[i]

LDRB R5, [R2], #1; load bit, y[i]

ADD R6, R4, R5; z[i] =x[i] + y[i]

ADD R6, R6, R7;  z[i] += c[i-1]

MOV R7, #0; c[i]=0

CMP R6, #9; ? z[i]>9

SUBHI R6, R6, #10; r6-10:-

MOVHI R7, #1;   r7=1:-

STRB R6, [R3], #1; z[i] -> mem[i]

ADD R0, R0, #1; i=i+1;

B loop; loop

done STRB R7, [R3]; storing carry

stop BAL stop

; input_number

x   DCB 1, 8, 0, 1, 2, 3, 4, 2;

y   DCB 6, 7, 5, 4, 5, 0, 4, 7;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory:

We can see that for our input 24321081 + 74054576 (in BCD)

=98375657, which matches our output.

7.5] Write a program to convert a given (Note: Consider BCD and HX to 4 digits no)

(a) ASCII to BCD

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

bcd SPACE 4;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =asc;

LDR R2, =cf;

;address to the result

LDR R3, =bcd ;

LDR R4, [R1]; load ascii val

LDR R5, [R2]; load conv factor

SUB R5, R4, R5;

STR R5, [R3]; storing quotient

stop BAL stop

; input_number

asc DCB "1212";

cf  DCD 0x30303030;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory:

We can see that output in hex(decimal) matches our input (ASCII).

(b) ASCII to HX

-> Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

bcd SPACE 4;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =asc;

LDR R2, =cf;

;address to the result

LDR R3, =bcd ;

MOV R8, #1;

MOV R7, #0;

MOV R10, #10;

LDRB R4, [R1, #3]; load ascii val

SUB R5, R4, #0x30; bcd = ascii-conv

ADD R7, R7, R5;

MUL R9, R8, R10;

LDRB R4, [R1, #2]; load ascii val

SUB R5, R4, #0x30; bcd = ascii-conv

MUL R5, R9, R5;

ADD R7, R7, R5;

MUL R8, R9, R10;

LDRB R4, [R1, #1]; load ascii val

SUB R5, R4, #0x30; bcd = ascii-conv

MUL R5, R8, R5;

ADD R7, R7, R5;

MUL R9, R8, R10;

LDRB R4, [R1]; load ascii val

SUB R5, R4, #0x30; bcd = ascii-conv

MUL R5, R9, R5;

ADD R7, R7, R5;

STR R7, [R3]; storing quotient

stop BAL stop

; input_number

asc DCB "1212";

cf  DCD 0x30303030;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory:

We can see that for the given input “1212” we got the proper hex value in the output.

(c) HX to ASCII

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for result

array SPACE 1024;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =inputnumber ;

;address for space used in operation

LDR R2, =array;

MOV R12, R2;

; load the  input number from memory

LDR R4, [R1];

LDRB R9, [R2];

ADD R2 ,#3;

; Getting the last bit

MOV R5, R4; R5 = R4

MOV R6, #0;

MOV R7, #0; for storing no of nos.

;dividing and finding nth digit.

;division by the use of subtracting

; until a remainder is found.

; where remainder < 10

div SUB R5, R5, #10; subtracting by 10

CMP R5, #10; checking if remainder < 10

;incrementing iterator to store quotient

ADD R6, R6, #1;

; if remainder >=10 then loop to sub further.

BPL div

ADD R7, #1; incrementing nos.

ADD R5, R5, #0x30;

STRB R5, [R2], #-1; storing digits

;comparing quotient and 10.

CMP R6, #10;

;if quotient >10 then store quotient in remainder

MOVPL R5, R6;

; && reset quotient

MOVPL R6, #0;

; && loop again

BPL div;

; check if quotient is zero

CMP R6, #0;

; increment no of digits counter if no

ADDNE R7, #1;

; &&  store it in R2 if no.

ADD R6, R6, #0x30;

STRBNE R6, [R2]; storing last digit

;

stop BAL stop

; input_number

inputnumber DCD 1425;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory: (in ascii mode)

We can see that our output matches the expected result for the input 1425.

(d) BCD to ASCII

->

Source Code:

AREA AllocSpace, DATA,NOINIT,READWRITE

;Space for storing result

;Utkarsh && Arnav Lab6

asc SPACE 4;

;Actual Code

AREA hmm, CODE, READWRITE

EXPORT Reset_Handler

Reset_Handler

;address to the input number

LDR R1, =bcd;

LDR R2, =cf;

;address to the result

LDR R3, =asc ;

LDR R4, [R1]; load ascii val

LDR R5, [R2]; load conv factor

ADD R5, R4, R5;

AND R6, R5, #0xFF;

AND R7, R5, #0xFF00;

AND R8, R5, #0xFF0000;

AND R9, R5, #0xFF000000;

LSL R6, #24; shifting towards other side as

LSL R7, #8; required for ascii string

LSR R8, #8;

LSR R9, #24;

MOV R5, R6;

ADD R5, R7;

ADD R5, R8;

ADD R5, R9;

STR R5, [R3]; storing quotient

stop BAL stop

; input_number

bcd DCB 2, 1, 2, 1;

cf  DCD 0x30303030;

END

Debugging:

Initial Memory: (after getting the address through register)

Setup:

Final Output:

Final Memory: