EC-340

Computer Organization and Architecture

Utkarsh Mahajan 201EC164

R S MUTHUKUMAR 201EC149

Devvrat Ashtaputre 201EC118

a) Assume that you have an array of 10 elements with base address in $s0. Write an assembly program to find the minimum value from the array and swap it with the last element in the array.

-> We will make use of word directives to store array into memory with base address labelled as array. For debugging the code, We will use la to store the base address at $s0 as asked to be assumed in the question. We will store index I in register $t1 initializing it to 0. And then check if it reaches 10 using slti. Through each iteration, we will first multiply index($t1) by 4 by left shifting by 2bits and store that offset value in $t6. Calculating respective index’s address by adding base + the offset.  Then through address we will check if it’s the minimum value until now using slt and if it is then we will jump to min where we will store the value  and its index in $t7 and $t0 respectively, we will increment at the end of the loop and then jump to loop label where the condition will be checked again.

Once the most minimum value has been found in the array after iterating through the array. Through beq, We will jump to swapmin. Here we will swap the minimum value with the last value of the array.

Later jumping to printl. Looping similarly, we will later print the updated array into the console using syscall with respective $v0 and $a0 values then jump to done which will jump to $ra (return address)

Assembly code:

.data

#  storing required data into memory

array:  .word 67 43 3 7 2 35 9 62 4 8

.text

.globl main

main:

li      $t1, 0              # i (index) = 0

la      $s0, array          # fetch base address

# initializing minimum =  storing a[0] value in minimum

lw      $t0, 0($s0)

li      $t7, 0              # index of minimum

# j (index) = 0 for printing updated array

li      $t9, 0

loop:   slti    $t3, $t1, 10         # if i == 10 goto done

beq     $t3, $zero, swapmin

sll     $t6, $t1, 2         # offset = index * 4

add     $t5, $s0, $t6       # address = base_address + offset;

lw      $t4, 0($t5)         # t4= arr[i]

slt     $t2, $t4, $t0       # setting less than in t2

# switching to min branch if a[i]<current_min

bne     $t2, $zero, min

# label b_loop for returning after min has been updated

b_loop: addi    $t1, $t1, 1         # i++

j       loop

# min updates the minimum value and the index containing it

min:    add     $t0, $zero, $t4     # updating minimum value

# updating index of minimum so that we can swap values letter

add     $t7, $zero, $t1

j      b_loop

# for printing the updated array

printl: slti    $t3, $t9, 10         # if i == 10 goto done

beq     $t3, $zero, done     # jumping to done if all elements are printed

sll     $t6, $t9, 2         # offset = index * 4

add     $t2, $s0, $t6       # address = base_address + offset;

lw      $t4, 0($t2)        # t4 = arr[i]

li      $v0, 1              # for printing

move    $a0, $t4            # print arr[i]

syscall

li      $v0, 11             # print space character

li      $a0, 32

syscall

addi    $t9, $t9, 1         # i++

# jump back to printl to print all values of 2nd array

j       printl

# for swapping the minimum and last array

swapmin:

lw      $t4,  36($s0)     # saving the last value of the array

sll     $t6, $t7, 2        # offset = min_index *4

sw      $t0, 36($s0)        # storing min value in last position

add     $t5, $s0, $t6       # min_address = base + offset

# storing the value of last element in the position of minimum value

sw      $t4, 0($t5)

j       printl          # to print the updated array

done:

jr $ra                      # return from main

Output:

Console output and input array value:

b) Assume that you have an array of 10 elements with base address in $s0. Assume that the base address of a second array is in $t0. Write an assembly program to copy the elements from the first array to the second.

->

We will make use of word directives to store first and second array labelling them as firstarray and secondarray respectively. For debugging the code, We will use la to store the base addresses at $s0  and $t0 respectively as asked to be assumed in the question. We will store index i in register $t1 initializing it to 0. And then check if it reaches 10 using slti.

Through each iteration, we will first multiply index($t1) by 4 by left shifting by 2bits and store that offset value in $t6. Calculating respective index’s address by adding base + the offset. We will store the element from first array to second array by loading a element with lw and storing into second array one with sw.  we will increment index at the end of the loop and then jump to loop label where the condition will be checked again.

Once the most minimum value has been found in the array after iterating through the array. Through beq, we will jump to swapmin. Here we will swap the minimum value with the last value of the array.

Later jumping to printl. Looping similarly, we will later print the updated array into the console using syscall with respective $v0 and $a0 values then jump to done which will jump to $ra (return address)

Assembly Code:

.data

#  storing required data into memory

firstarray:  .word 65 12 56 145 41 311 425 601 48 895264

secondarray:  .word 4 0 0 0 3 0 0 0 0 5

.text

.globl main

main:

li      $t1, 0              # i (index) = 0

li      $t7, 0              # j (index) = 0 for print

la      $s0, firstarray     # fetch base address

# initializing minimum =  storing a[0] value in minimum

la      $t0, secondarray

# label for looping throughtout the array and copying to second array

loop:   slti    $t3, $t1, 10         # if i == 10 goto done

# after completition, jump to printl to print 2nd array

beq     $t3, $zero, printl

sll     $t6, $t1, 2         # offset = index * 4

add     $t5, $s0, $t6       # address_1 = base_address + offset;

lw      $t4, 0($t5)         # t4= memory[address_1]  /// t4 = arr_1[i]

add     $t2, $t0, $t6       # address_2 = base_address + offset;

sw      $t4, 0($t2)        # memory[address_2] = t4 /// arr_2[i]= t4

addi    $t1, $t1, 1         # i++

j       loop

# for printing the second array

printl: slti    $t3, $t7, 10         # if i == 10 goto done

beq     $t3, $zero, done

sll     $t6, $t7, 2         # offset = index * 4

add     $t2, $t0, $t6       # address_2 = base_address + offset;

lw      $t4, 0($t2)        # t4 = arr_2[i]

li      $v0, 1              # for printing

move    $a0, $t4            # print arr_2[i]

syscall

li      $v0, 11             # print space character

li      $a0, 32

syscall

addi    $t7, $t7, 1         # i++

# jump back to printl to print all values of 2nd array

j       printl

done:

jr      $ra               # return from main

Output:

Console output and input array value:

c) Write an assembly program to convert red-green-blue (RGB) values for a set of pixels into a single gray value per pixel. You are given an array called pixels, each element of which is a 32-bit word representing a color value. The lowest 8 bits of each color value denote an unsigned integer representing the BLUE value, the next 8 bits are the GREEN value, the next 8 bits are the RED value, and the most significant 8 bits are all zeroes. gray value = (red + green + blue) / 3 (integer divide and truncate). Use a separate procedure rgb2gray and print each RGB value and the corresponding gray value on the console.

-> Similar to the codes above, we will store the needed values into memory, here for debugging under .data section using data directives like .asciiz (for strings) and .word.

We will use arguments $a2 and $a3 for indexing. Store base address for RGB input array in $s0 and for output gray array in $s1 .

Since we will be making a procedure call, we will need to save the return address of the main.  For that we will allocate 32 bit space by decrementing stack pointer($sp) by 4(in bytes). And store the return address in it using lw.

We will jump to the procedure rgb2gray using function jal which will update the $ra (return address).  Inside the rgb2gray. We will loop until index i( stored in $a3) reaches 10. We will increment I at every end of the loop.

In the loop,  At first we will calculate offset by multiplying index by 4 (left shifting by 2 bits) and storing it in a temporary variable $t2. Then for getting the i’th element of the rgb array. we will calculate its respective address by adding base($s0)  and offset($t2) and store it in $t4. Loading the rgb value in $t5. We will use andi with immediate #0xff, to get the lowest significant byte of it which is the value of blue and store it in a temporary variable $t6. Later right shifting $t5 by a byte. We get Green value at Lowest significant byte. This can again be separated using andi, and then will be added to $t6. Repeating the right shift, we will have red value at lowest significant byte. Which will be separated similarly using andi. Then added to $t6.

We will have all the R, G and B values summed up in $t6. We will divide it by 3 using div and store the quotient in $t6 itself. Later to store the value from the temporary register $t6 into the respective I’th element of gray code’s array. by calculating address for the I’th element similar to what we did for RGB array elements. But here we will store the value (using sw) and not load.

After Gray code values of all the 10 RGB code values have been calculated, we will jump to printl. Where with the help of syscall and similar looping, We will print all the RGB code and its gray code respectively.  After the looping it will jump to done which will return to the next address of where the rgb2gray was called. Here, the main’s return address is restored in $ra return address

From stack pointer and then the space allocated for it is deallocated by incrementing the stack pointer. After it, the main function will return and the program ends.

Assembly Code:

.data

#  storing required data into memory

rgb:  .word 0xbf49ac 0x68dba6 0xFF0A5D 0x06c012 0x1428e5 0x1ec2b2 0x5e7386 0xae770f 0x286dd0 0x8b10b6

gray:  .word 4 0 0 0 3 0 0 0 0 5

#  storing strings required to print at the end

trgb:   .asciiz "RGB: "

tgray:  .asciiz " GRAY: "

.text

.globl main

main:

li      $a2, 0              # i (index) = 0

la      $s0, rgb     # fetch base address

# initializing minimum =  storing a[0] value in minimum

la      $s1, gray

li      $s3, 3       # to store 3 to divide later

li      $a3, 0       # j index =0 for printing

addi    $sp, $sp, -4 # allocating space on stack

sw      $ra, 0($sp)  # storing main's return address

jal     rgb2gray     # jumping (with link) to rgb2gray function

lw      $ra, 0($sp)  # loading back main's return address

addi    $sp, $sp, 4  # deallocating space on stack

jr      $ra          # returning from main. END OF PROGRAM

# procedure(Function) rgb2gray

rgb2gray:   slti    $t3, $a2, 10         # if i == 10 goto done

beq     $t3, $zero, printl

sll     $t2, $a2, 2         # offset = index (i) * 4

# address_1 = base_address + offset;

add     $t4, $s0, $t2

# t5= memory[address_1]  /// t5 = rgb[i]

lw      $t5, 0($t4)

# t6 = rgb[i][7:0] = blue value // last 7 bits by using AND

andi    $t6, $t5, 0xFF

# right shift t5 by 8 so GREEN comes at lowest significant byte.

srl     $t5, $t5, 8

# t7 = rgb[i][7:0] = green value  // last 7 bits by using AND

andi    $t7, $t5, 0xFF

# t6 = t7 + t6 // t6 = blue + green

add     $t6, $t7, $t6

# right shift t5 by 8 so RED comes at lowest significant byte.

srl     $t5, $t5, 8

# t7 = t5[i][7:0] = green value  // last 7 bits by using AND

andi    $t7, $t5, 0xFF

# t6 = t7 + t6 // t6 = blue + green + red

add     $t6, $t7, $t6

# t6 = t6 / t8 = (blue + green + red) /3

divu    $t6, $t6, $s3

# address_2 = base_address + offset;

add     $t4, $s1, $t2

#  memory[address_1]= t6  /// gray[i] = t6 = (r+g+b)/3

sw      $t6, 0($t4)

addi    $a2, $a2, 1         # i++

j       rgb2gray

# for printing RGB values and its respective Gray values

printl: slti    $t3, $a3, 10         # if i == 10 goto done

beq     $t3, $zero, done

sll     $t6, $a3, 2         # offset = index * 4

# rgb_address = base_rgb_address + offset;

add     $t2, $s0, $t6

lw      $t4, 0($t2)        # t4 = rgb[i]

li      $v0, 4

la      $a0, trgb            # printing string "RGB: "

syscall

li      $v0, 1              # for printing

move    $a0, $t4            # print rgb[i]

syscall

li      $v0, 4

la      $a0, tgray           # printing string " GRAY: "

syscall

# gray_address = base_gray_address + offset;

add     $t2, $s1, $t6

lw      $t4, 0($t2)        # t4 = gray[i]

li      $v0, 1              # for printing

move    $a0, $t4            # print gray[i]

syscall

li      $v0, 11             # print newline character

li      $a0, 0x0a

syscall

addi    $a3, $a3, 1         # i++

# jump back to printl to print all values of 2nd array

j       printl

done:

jr      $ra   # return from rgb2gray to main

Output:

Console output and input array value:

Note: The console displays values in decimal values and not in hex.